Content

2	Classes	2

2.1	AirInterfaceCapability	2

2.2	AirInterfaceConfiguration	6

2.3	AirInterfaceCurrentPerformance	13

2.4	AirInterfaceHistoricalPerformances	14

2.5	AirInterfaceLpSpec	15

2.6	AirInterfaceStatus	15

2.7	AirInterface_Pac	20

2.8	CurrentPerformance	21

2.9	HistoricalPerformance	23

2.10	TransmissionMode	24

3	Data Types	27

3.1	AcmThresholdCrossAlarmType	27

3.2	AirInterfaceCurrentPerformanceType	28

3.3	AirInterfaceHistoricalPerformanceType	29

3.4	AirInterfacePerformanceType	29

3.5	G826ThresholdCrossAlarmType	34

3.6	RadioSignalIdType	35

3.7	TimeXStatesType	36

3.8	XltsThresholdCrossAlarmType	37

4	Enumeration Types	38

4.1	DirectionType	38

4.2	G826Type	38

4.3	GranularityPeriodType	39

4.4	InterfaceStatusType	39

4.5	LayerProtocolNameType	40

4.6	LoopBackType	40

4.7	PolarizationType	40

4.8	RadioSignalIdDatatypeType	40

4.9	XLevelThresholdSecondKindType	41

5	Primitive Types	41

# Classes

## AirInterfaceCapability

Describes the 'analog' capabilities of modem and transmitter of the microwave device. Value ranges of attributes are not independently (e.g. min. and max. transmit power depends on modulation). Legal combinations of values are expressed in transmissionModeTypes.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceCapability

Table 1: Attributes for AirInterfaceCapability

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| typeOfEquipment | String Type of equipment not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | This parameter indicates the equipment type. Instead of uploading the complete set of capabilities, capabilities of the same equipment type could be reused. Should be unique for a combination of modem, radio and their respective firmware. |
| txFrequencyMin | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Value of the minimum transmit frequency tunable at the air interface. |
| txFrequencyMax | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Value of the maximum transmit frequency tunable at the air interface. |
| rxFrequencyMin | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Value of the minimum receive frequency tunable at the air interface. |
| rxFrequencyMax | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Value of the maximum receive frequency tunable at the air interface. |
| duplexDistanceIsFreelyConfigurable | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = Transmitted and received frequency can be freely chosen between the given minimum and maximum values. |
| duplexDistanceList | Integer -1 | 0..* | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Only relevant if (duplexDistanceIsFreelyConfigurable==false). Lists all supported distances between transmitted and received frequency. |
| _transmissionModeList | TransmissionMode ./. | 1..* | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of supported Transmission Modes. |
| autoFreqSelectIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | In case the microwave radio is capable of automatically selecting the transmit frequency in unlicensed bands, this field shall contain a 'true'. |
| receiverOnOffIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | In case switching on/off the receiver is available at the device, this field shall contain a 'true'. |
| adaptiveModulationIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | In case the device is capable of adaptive modulation, this field shall contain a 'true'. |
| atpcIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | In case the microwave radio is capable of ATPC, this field shall contain a 'true'. |
| atpcRange | Integer 0 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dB • support: MANDATORY | Extent of the ATPC range. This value represents a device specific maximum value. The actual range of the ATPC at a specific link might be limited by the difference between configured transmit power (AirInterface::AirInterfaceConfiguration::txPower) and minimum transmit power of the device (TypeDefinitions::TransmissionModeType::txPowerMin). |
| supportedRadioSignalIdDatatype | RadioSignalIdDatatypeType NOT_YET_DEFINED | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | This attribute is for expressing the datatype, which is supported as a Radio Signal ID by the device. |
| supportedRadioSignalIdLength | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If (supportedRadioSignalIdDatatype==INTEGER): Maximum value of the Radio Signal ID supported by the device; If (supportedRadioSignalIdDatatype==STRING): Maximum number of letters supported by the device. |
| expectedEqualsTransmittedRadioSignalID | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = It is not possible to configure an expectedRadioSignalID different from the transmittedRadioSignalID. |
| encryptionIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Shall be marked 'true', if payload encryption is available. |
| supportedLoopBackKindList | LoopBackType ./. | 1..4 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of supported kinds of looping back of header information to the remote site. |
| maintenanceTimerRange | String Range of the maintenance timer not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Available time periods for maintenance configurations (e.g. the loop back of microwave header information) to be described. Concrete values shall be separated by commas (e.g. '10, 60, 360'). Ranges shall be expressed as two values separated by a minus (e.g. '10-360'). Values shall be expressed in seconds. The time unit shall not be mentioned in the string. |
| acmThresholdCrossAlarmsIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = Threshold cross alarms on performance of adaptive code modulation are available. |
| clearingThresholdCrossAlarmsIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = Clearing threshold cross alarms by calling clearThresholdCrossAlarms() operation is available at the device. |
| performanceMonitoringIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = Collection and aggregation of performance values is available. |
| directionOfAcmPerformanceValues | DirectionType NONE | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Expresses, whether the ACM performance values (timeXStatesList) are measured at the transmitter (TX) or at the receiver (RX). |

## AirInterfaceConfiguration

Configuration of the radio link.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceConfiguration

Table 1: Attributes for AirInterfaceConfiguration

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| airInterfaceName | String Air interface ID not yet defined. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Operator specific microwave link ID (often used for coding area, type of element and sequential number). |
| remoteAirInterfaceName | String Air interface ID at the remote site not yet defined. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Name of the air interface, which belongs to the same link, at the remote site. |
| transmittedRadioSignalID | RadioSignalIdType ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Transmitted radio signal ID for synchronizing the remote receiver. |
| expectedRadioSignalID | RadioSignalIdType ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Only relevant, if expectedEqualsTransmittedRadioSignalID==false. Received radio signal ID for identifying the correct transmitter to synchronize on. |
| txFrequency | Integer -1 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: kHz • support: MANDATORY | Center frequency of the transmit channel. The values to be configured have to exactly match the values listed in the international agreement referenced in channelPlanID. In case of automated selection of the transmit frequency this field shall describe the lowest center frequency selectable. |
| rxFrequency | Integer -1 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: kHz • support: MANDATORY | Only configurable, if (duplexDistanceIsFreelyConfigurable==true). Allows configuring of the center frequency of the receive channel. |
| duplexDistance | Integer -1 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: kHz • support: MANDATORY | Only configurable, if (duplexDistanceIsFreelyConfigurable==false) AND (duplexDistanceList contains more than one value). Allows configuring one of the values represented in AirInterfaceCapability::duplexDistanceList. |
| _transmissionModeMin | TransmissionMode ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY PassedByReference | Minimum transmission mode to be configured (in case adaptive modulation is not used, this value represents also the fixed transmission mode). |
| _transmissionModeMax | TransmissionMode ./. | 0..1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY PassedByReference | Maximum transmission mode to be configured. |
| powerIsOn | Boolean true | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = The power to all physical components required for this AirInterface instance is turned on. |
| transmitterIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activation of the transmitter inside the radio shall be expressed as a 'true'. |
| receiverIsOn | Boolean true | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Maintenance Feature. Activation of the receiver inside the radio shall be expressed as a 'true'. Attribute shall also be used for RX main and RX diversity squelches in case of diversity configurations. |
| txPower | Integer 99 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | Transmit power to be configured on the microwave link. Signed Byte is required. The actually operated transmit power might be lower depending on adaptive modulation and ATPC. |
| adaptiveModulationIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Adaptive Modulation. Activation of adaptive modulation shall be expressed as a 'true'. |
| xpicIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activation of Cross Polarization Interference Cancelation shall be expressed as a 'true'. In case XPIC is not available for the current combination of channel bandwidth and modulation or the hardware in general, this parameter shall always be set to 'false'. |
| mimoIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activation of Multiple Input Multiple Output (MIMO) shall be expressed as a 'true'. |
| alicIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activation of Adjacent Link Interference Cancelation (ALIC) shall be expressed as a 'true'. |
| atpcIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | ATPC. Activation of Automated Transmit Power Control shall be expressed as a 'true'. |
| atpcThreshUpper | Integer 99 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | If the receive level at the local/remote site is higher than the upper threshold value, the transmitter at the remote/local site is notified to decrease transmit power. In case the device does not know upper and lower threshold values, but only a single reference value, the atpcThreshUpper shall not be available for configuration and constantly represent the default value. |
| atpcThreshLower | Integer 99 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | If the receive level at the local/remote site is lower than the lower threshold value, the transmitter at the remote/local site is notified to increase transmit power. In case the device does not know upper and lower threshold values, but only a single reference value, the atpcThreshLower shall represent this reference value. |
| atpcTxPowerMin | Integer -99 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | Transmit power, which is not to be undercut, while operating ATPC. |
| autoFreqSelectIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activation of automatically selecting the transmit frequency in unlicensed bands shall be expressed as a 'true'. |
| autoFreqSelectRange | Integer -1 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: channels • support: MANDATORY | Number of transmit channels (starting at the center frequency defined in txFrequency and with channel bandwidth according to txChannelBandwidth) that define the range within the transmit frequency can automatically been chosen. |
| modulationIsOn | Boolean true | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Maintenance Feature. De-activation of the modulation of the carrier signal for fault management shall be expressed as a 'false'. |
| encryptionIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Activates encryption of the payload. |
| cryptographicKey | String Cryptographic key not yet defined. | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Key for transforming plaintext into ciphertext data. |
| loopBackKindOn | LoopBackType NONE | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Maintenance Feature. The currently configured type of looping back of the air interface header shall be expressed here. The received header is returned to the remote site. |
| maintenanceTimer | Integer -1 | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: s • support: MANDATORY | Time of existence of any maintenance configuration (e.g. the loop back of microwave header information). 0 = maintenance timer is switched off. Valid values are defined in AirInterface::AirInterfaceCapability::maintenanceTimerRange. |
| g826ThresholdCrossAlarmList | G826ThresholdCrossAlarmType ./. | 0..* | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of G826 related threshold cross alarms to be configured. |
| xltsThresholdCrossAlarmList | XltsThresholdCrossAlarmType ./. | 0..* | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List of RLTS or TLTS (ETSI EN 301 129) related threshold cross alarms to be configured. |
| acmThresholdCrossAlarmList | AcmThresholdCrossAlarmType ./. | 0..* | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | List threshold cross alarms, which relate to the time period of operation of a specific transmission mode. |
| performanceMonitoringIsOn | Boolean false | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Enables measurement, collection, storage and access to performance data. |
| polarization | PolarizationType NOT_YET_DEFINED | 1 | RW | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Allows the operator to document the polarization of the AirInterface. |

## AirInterfaceCurrentPerformance

Aggregated performance information of the air interface at a particular moment.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceCurrentPerformance

Table 1: Attributes for AirInterfaceCurrentPerformance

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| currentPerformanceDataList | AirInterfaceCurrentPerformanceType ./. | 1..2 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | At least values of the counters, which are reset every 15 minutes, are to be provided. If available, the current values of the counters, which are reset every 24 hour, can be provided, too. |
| numberOfCurrentPerformanceSets | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Number of sets of current performance values, which are provided in the list. |

## AirInterfaceHistoricalPerformances

Aggregated performance information of the air interface for a pre-defined measurement interval.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceHistoricalPerformances

Table 1: Attributes for AirInterfaceHistoricalPerformances

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| historicalPerformanceDataList | AirInterfaceHistoricalPerformanceType ./. | 0..* | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY |  |
| numberOfHistoricalPerformanceSets | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Number of sets of historical performance values, which are provided in the list. |
| timeOfLatestChange | DateTime 2010-11-20T14:00:00+01:00 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Date and time when the list of sets of historical performance values has been changed for the last time (e.g. new one added or existing one deleted). |

## AirInterfaceLpSpec

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: YES

- objectDeletionNotification: YES

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceLpSpec

Table 1: Attributes for AirInterfaceLpSpec

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _airInterface_Pac | AirInterface_Pac ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## AirInterfaceStatus

Measurements of current values on the air interface and operational status of the device.

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterfaceStatus

Table 1: Attributes for AirInterfaceStatus

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| interfaceStatus | InterfaceStatusType NOT_YET_DEFINED | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Operational status of the interface. |
| txFrequencyCur | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: kHz • support: MANDATORY | Center frequency of the currently operated transmit channel. |
| rxFrequencyCur | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: kHz • support: MANDATORY | Center frequency of the currently operated receive channel. |
| _transmissionModeCur | TransmissionMode ./. | 0..1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY PassedByReference | Currently operated transmission mode according to definitions in Capabilities. |
| receivedRadioSignalID | RadioSignalIdType ./. | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Radio signal ID of the signal, which the receiver is currently synchronized on. |
| linkIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If connection is established to the remote site with the same linkID, this shall be expressed as a 'true'. |
| xpicIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If XPIC is currently actually working (not just configured), this shall be expressed as a 'true'. |
| mimoIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If MIMO is currently actually working (not just configured), this shall be expressed as a 'true'. |
| alicIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If Adjacent Link Interference Cancelation (ALIC) is currently actually working (not just configured), this shall be expressed as a 'true'. |
| atpcIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If ATPC is currently actually working (not just configured), this shall be expressed as a 'true'. |
| autoFreqSelectIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | If automated frequency selection is currently actually working (not just configured), this shall be expressed as a 'true'. |
| localEndPointId | String not-supported | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | The value of the localEndPointId is a vendor specific identifier of the air interface, used by the node to discover a microwave radio link. |
| remoteEndPointId | String not-supported | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | The value of the remoteEndPointId is a vendor specific identifier of the airinterface at the remote side, used to by the node to discover a microwave radio link. |
| loopBackKindUp | LoopBackType NONE | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | The currently active (not just configured) type of looping back of the air interface header shall be expressed here. The received header is returned to the remote site. |
| performanceMonitoringIsUp | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: YES • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = Performance values are currently collected and aggregated. |
| rxLevelCur | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | Current receive level. |
| txLevelCur | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dBm • support: MANDATORY | Current transmit level. |
| snirCur | Integer -99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dB • support: MANDATORY | Currently measured signal to (noise+interference) ratio. |
| xpdCur | Integer -99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: dB • support: MANDATORY | Currently measured cross polarization discrimination. |
| rfTempCur | Integer -99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: false • valueRange: no range constraint • unit: Celsius • support: MANDATORY Deprecated | Current temperature (in degree Celsius) of the radio module inside the outdoor unit. |

## AirInterface_Pac

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for AirInterface_Pac

Table 1: Attributes for AirInterface_Pac

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _airInterfaceCapability | AirInterfaceCapability ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _airInterfaceConfiguration | AirInterfaceConfiguration ./. | 1 | RW | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _airInterfaceStatus | AirInterfaceStatus ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _airInterfaceCurrentPerformance | AirInterfaceCurrentPerformance ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |
| _airInterfaceHistoricalPerformances | AirInterfaceHistoricalPerformances ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | See referenced class |

## CurrentPerformance

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for CurrentPerformance

Table 1: Attributes for CurrentPerformance

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| timestamp | DateTime 2010-11-20T14:00:00+01:00 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | The timestamp associated with when the current data was collected. |
| suspectIntervalFlag | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | This attribute is used to indicate that the performance data for the current period may not be reliable. Some reasons for this to occur are: - Suspect data were detected by the actual resource doing data collection. - Transition of the administrativeState attribute to/from the 'lock' state. - Transition of the operationalState to/from the 'disabled' state. - Scheduler setting that inhibits the collection function. - The performance counters were reset during the interval. - The currentData (or subclass) object instance was created during the monitoring period. |
| elapsedTime | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_64_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: s • support: MANDATORY | Number of seconds that elapsed since the last reset of the counter. |
| scannerId | String Scanner ID not defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY |  |
| granularityPeriod | GranularityPeriodType ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 1 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Time period between reset of the underlying counter. |

## HistoricalPerformance

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for HistoricalPerformance

Table 1: Attributes for HistoricalPerformance

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| suspectIntervalFlag | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | This attribute indicates that the data collected during the interval is suspect. |
| historyDataId | String History Data ID not defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY |  |
| granularityPeriod | GranularityPeriodType ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 1 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Time period between reset of the underlying counter. |
| periodEndTime | DateTime ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 2 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Time when the counter values have been recorded and the counter reset. |

## TransmissionMode

Applied stereotypes:

- OpenInterfaceModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- OpenModelClass

- support: MANDATORY

Attributes for TransmissionMode

Table 1: Attributes for TransmissionMode

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| transmissionModeName | String ./. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 1 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Name of the transmission mode. BBBB-m*-i*/t*-r*. B=four digits of channel bandwidth in MHz. m*=required number of digits for modulation name. (i*/t*=code rate.) i*=required number of digits for number of information bits. t*=required number of digits for total bits. r*=required number of digits for rate reduction factor. Example: 0028-4QAM-188/204-1 |
| transmissionModeRank | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Configuring adaptive modulation by selecting _transmissionModeMin and _transmissionModeMin requires the transmission modes to be ordered. The vendor shall rank all available transmission modes according to the respective payload data rate. The application providers shall order the transmission modes according to the rank. |
| channelBandwidth | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_32_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: kHz • support: MANDATORY | Bandwidth of the transmit channel. The value shall be expressed explicitly (means in kHz) not as a reference to an international agreement. The values shall be chosen from the following _list: 3.500, 7.000, 14.000, 27.500, 28.000, 29.000, 29.650, 30.000, 40.000, 50.000, 55.000, 56.000, 59.300, 60.000, 80.000, 100.000, 112.000, 120.000, 150.000, 200.000, 250.000, 500.000, 750.000, 1.000.000, 1.250.000, 1.500.000, 1.750.000, 2.000.000; |
| modulationScheme | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: symbols • support: MANDATORY | Modulation scheme, which is base to the other characteristics described in the same transmissionModeType data type. The modulation scheme shall be described by the number of states in the phase diagram (e.g. BPSK->'2' or 256QAM->'256'). |
| codeRate | Integer -1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: % • support: MANDATORY | Code rate of the coding scheme in % (Net bit rate is either smaller or equal to (Gross bit rate mulitplied with code rate) ). |
| symbolRateReductionFactor | Integer 1 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Reduction factor for the symbol rate. Example: value would be 4 for 1/4BPSK. |
| txPowerMin | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dBm • support: MANDATORY | Value of the minimum transmit power the modem can operate in dBm. |
| txPowerMax | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dBm • support: MANDATORY | Value of the maximum transmit power the modem can operate in dBm. |
| rxThreshold | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_16_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dBm • support: MANDATORY | Value of the receive level required to decode the received signal with a Bit Error Rate of 1e-6 or less. |
| amUpshiftLevel | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dBm • support: MANDATORY | Value of the receive level that has to be exceeded to shift into a higher modulation scheme. |
| amDownshiftLevel | Integer 99 | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: LENGTH_8_BIT OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: dBm • support: MANDATORY | Value of the receive level that has to be exceeded for not shifting into a lower modulation scheme. |
| xpicIsAvail | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | In case this transmission mode is capable of XPIC, this attribute shall contain a 'true' value. This information shall strictly relate to the capability of setting an administrative state (xpic-is-on attribute could be configured to have a 'true' value) and is independent of a potential instantaneous operational status change. For example, this attribute might contain a 'true' value, even if an additional cable would have been required to actually operate XPIC at a specific microwave link. |
| supportedAsFixedConfiguration | Boolean false | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | true = This transmission mode is available for manual configuration when Adaptive Modulation is switched off. |
| modulationSchemeNameAtLct | String Modulation scheme name not yet defined. | 1 | R | OpenInterfaceModelAttribute • AVC: NO • bitLength: NA OpenModelAttribute • partOfObjectKey: 0 • isInvariant: true • valueRange: no range constraint • unit: no unit defined • support: MANDATORY | Modulation scheme name, which is together with the applied channel bandwidth identifying this transmission mode at the graphical user interface of the local craft terminal. |

# Data Types

## AcmThresholdCrossAlarmType

Applied Stereotypes:

Attributes for AcmThresholdCrossAlarmType

Table 1: Attributes for AcmThresholdCrossAlarmType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| acmThresholdCrossAlarmDefinitionNumber | Integer ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 1 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Sequence number of the definitions. |
| _transmissionMode | TransmissionMode ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: no unit defined - support: MANDATORY PassedByReference |  |
| secondsForRaisingAlarm | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: s - support: MANDATORY | Number of seconds, in which the referenced transmission mode has to be operated until the threshold cross alarm is raised for this 15min or 24hour period. |
| secondsForClearingAlarm | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: s - support: MANDATORY | Number of seconds, in which the referenced transmission mode has to be operated until the threshold cross alarm is cleared for this 15min or 24hour period. |
| granularityPeriod | GranularityPeriodType ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 2 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Period of the performance data collection. |

## AirInterfaceCurrentPerformanceType

Turns performance information into current performance information by inheriting from OTN_CurrentData.

Applied Stereotypes:

Attributes for AirInterfaceCurrentPerformanceType

Table 1: Attributes for AirInterfaceCurrentPerformanceType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| performanceData | AirInterfacePerformanceType ./. | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY |  |

## AirInterfaceHistoricalPerformanceType

Turns performance information into historical performance information by inheriting from OTN_HistoryData.

Applied Stereotypes:

Attributes for AirInterfaceHistoricalPerformanceType

Table 1: Attributes for AirInterfaceHistoricalPerformanceType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| performanceData | AirInterfacePerformanceType ./. | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY |  |

## AirInterfacePerformanceType

Consolidated performance information of the air interface.

Applied Stereotypes:

Attributes for AirInterfacePerformanceType

Table 1: Attributes for AirInterfacePerformanceType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| es | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Number of errored seconds. |
| ses | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Number of severely errored seconds. |
| cses | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Number of consecutive severely errored seconds. |
| unavailability | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Total time of unavailability in seconds. |
| txLevelMin | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Minimum transmit power. Signed integers are required. |
| txLevelMax | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Maximum transmit power. Signed integers are required. |
| txLevelAvg | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Averaged transmit power. Signed integers are required. |
| rxLevelMin | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Minimum receive level. Signed integers are required. |
| rxLevelMax | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Maximum receive level. Signed integers are required. |
| rxLevelAvg | Integer 99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dBm - support: MANDATORY | Averaged receive level. Signed integers are required. |
| timeXStatesList | TimeXStatesType ./. | 1..* | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Time period the transmitter operated in the respective transmission mode. |
| snirMin | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Minimum signal to (noise+interference) ratio. |
| snirMax | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Maximum signal to (noise+interference) ratio. |
| snirAvg | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Averaged signal to (noise+interference) ratio. |
| xpdMin | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Minimum cross polarization discrimination. |
| xpdMax | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Maximum cross polarization discrimination. |
| xpdAvg | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: dB - support: MANDATORY | Averaged cross polarization discrimination. |
| rfTempMin | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: Celsius - support: MANDATORY | Lowest temperature (in degree Celsius) of the radio module inside the outdoor unit. |
| rfTempMax | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: Celsius - support: MANDATORY | Highest temperature (in degree Celsius) of the radio module inside the outdoor unit. |
| rfTempAvg | Integer -99 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: Celsius - support: MANDATORY | Averaged temperature (in degree Celsius) of the radio module inside the outdoor unit. |
| defectBlocksSum | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_16_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: blocks - support: MANDATORY | Total number of blocks that were defect after receiving and could not be corrected by the FEC. |
| timePeriod | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Total length of the measurement period. |

## G826ThresholdCrossAlarmType

Allows defining a threshold cross alarm.

Applied Stereotypes:

Attributes for G826ThresholdCrossAlarmType

Table 1: Attributes for G826ThresholdCrossAlarmType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| g826ValueKind | G826Type ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 1 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Kind of performance value that shall be equipped with a threshold alarm. |
| alarmRaisingThreshold | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: s - support: MANDATORY | Number of events required for raising the threshold cross alarm. |
| alarmClearingThreshold | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: s - support: MANDATORY | Number of events required for clearing the threshold cross alarm. |
| granularityPeriod | GranularityPeriodType ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 2 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Period of the performance data collection. |

## RadioSignalIdType

Applied Stereotypes:

Attributes for RadioSignalIdType

Table 1: Attributes for RadioSignalIdType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| numericRadioSignalID | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Only relevant if (supportedRadioSignalIdDatatype==INTEGER): 0 = the receiver ignores the signal ID of the received signal. Any other value = the receiver exclusively synchronizes on signals with the same signal ID. -1 = to be used, if numericRadioSignalID is not supported or for expectedRadioSignalID, if it has to equal transmittedRadioSignalID. |
| alphanumericRadioSignalID | String Not yet defined. | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Only relevant if (supportedRadioSignalIdDatatype==STRING): Empty String = the receiver synchronizes on the received signal regardless of the signal ID of the received signal. Any other entry = the receiver exclusively synchronizes on signals with the same signal ID. Not yet defined. = to be used, if alphanumericRadioSignalID is not supported or for expectedRadioSignalID, if it has to equal transmittedRadioSignalID. |

## TimeXStatesType

Applied Stereotypes:

Attributes for TimeXStatesType

Table 1: Attributes for TimeXStatesType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| timeXStateSequenceNumber | Integer ./. | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 1 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Sequence number of the performance value. |
| _transmissionMode | TransmissionMode ./. | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY PassedByReference | Operated transmission mode. |
| time | Integer -1 | 1 | R | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: true - valueRange: no range constraint - unit: s - support: MANDATORY | Sum of all seconds the transmitter operated the transmission mode. |

## XltsThresholdCrossAlarmType

Applied Stereotypes:

Attributes for XltsThresholdCrossAlarmType

Table 1: Attributes for XltsThresholdCrossAlarmType

| Attribute Name | Type | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| levelThresholdSecondKind | XLevelThresholdSecondKindType ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 1 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Defines whether receive (RLTS) or transmit (TLTS) level triggers the threshold cross alarm. |
| xltsLevel | Integer 99 | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: dBm - support: MANDATORY | If (LevelThresholdSecondKindType==RLTS): Receive level, which has to be undercut, for the performance counter incrementing the number of seconds. If (LevelThresholdSecondKindType==TLTS): Transmit level, which has to be exceeded, for the performance counter incrementing the number of seconds. See also ETSI EN 301 129 V1.1.2 (1999-05). |
| amountOfSeconds | Integer -1 | 1 | RW | OpenInterfaceModelAttribute - AVC: YES - bitLength: LENGTH_32_BIT OpenModelAttribute - partOfObjectKey: 0 - isInvariant: false - valueRange: no range constraint - unit: s - support: MANDATORY | Number of seconds, which has to be exceeded for causing the threshold cross alarm to raise for this period. |
| granularityPeriod | GranularityPeriodType ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: NA OpenModelAttribute - partOfObjectKey: 2 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Period of the performance data collection. |
| xltsThresholdCrossAlarmDefinitionNumber | Integer ./. | 1 | RW | OpenInterfaceModelAttribute - AVC: NO - bitLength: LENGTH_8_BIT OpenModelAttribute - partOfObjectKey: 3 - isInvariant: true - valueRange: no range constraint - unit: no unit defined - support: MANDATORY | Sequence number of the definitions. |

# Enumeration Types

## DirectionType

Contains Enumeration Literals:

- TX:

- RX:

- NONE:

## G826Type

Contains Enumeration Literals:

- ES:

  - Errored Seconds. Threshold cross alarm will relate to TypeDefinitions::AirInterfacePerformanceType::es .

- SES:

  - Severely Errored Seconds. Threshold cross alarm will relate to TypeDefinitions::AirInterfacePerformanceType::ses .

- CSES:

  - Consecutive Severely Errored Seconds. Threshold cross alarm will relate to TypeDefinitions::AirInterfacePerformanceType::cses .

- UAS:

  - Unavailable Seconds. Threshold cross alarm will relate to TypeDefinitions::AirInterfacePerformanceType::unavailability.

- NOT_SPECIFIED:

## GranularityPeriodType

The enumeration with the options for granularity period of the performance data.

Contains Enumeration Literals:

- UNKNOWN:

- PERIOD-15-MIN:

- PERIOD-24-HOURS:

- NOT_YET_DEFINED:

## InterfaceStatusType

Current Interface Status

Contains Enumeration Literals:

- UP:

  - Ready to pass packets. It is expected that the LogicalTerminationPoint::operationalState attribute is expressing this logical layer being available for use (means: ENABLED), while this value occurs.

- DOWN:

  - The interface does not pass any packets. It is expected that the LogicalTerminationPoint::operationalState attribute is expressing this logical layer being NOT available for use (means: DISABLED), while this value occurs.

- TESTING:

  - In some test mode.  No operational packets can be passed. It is expected that the LogicalTerminationPoint::operationalState attribute is expressing this logical layer being NOT available for use (means: DISABLED), while this value occurs.

- UNKNOWN:

  - Status cannot be determined for some reason. While this value occurs, the LogicalTerminationPoint::operationalState attribute, which is expressing the availability of the logical layer for being used, might have either ENABLED or DISABLED as value.

- DORMANT:

  - Waiting for some external event. It is expected that the LogicalTerminationPoint::operationalState attribute is expressing this logical layer being available for use (means: ENABLED), while this value occurs.

- NOT_PRESENT:

  - Some component (typically hardware) is missing. It is expected that the LogicalTerminationPoint::operationalState attribute is expressing this logical layer being NOT available for use (means: DISABLED), while this value occurs.

- NOT_YET_DEFINED:

  - While this value occurs, the LogicalTerminationPoint::operationalState attribute, which is expressing the availability of the logical layer for being used, might have either ENABLED or DISABLED as value.

## LayerProtocolNameType

A controlled list of LayerProtocol names.

Contains Enumeration Literals:

- LAYER_PROTOCOL_NAME_TYPE_AIR_LAYER:

## LoopBackType

Contains Enumeration Literals:

- RF_TO_REMOTE:

  - Returning the header information of the remote site back to the remote site on the radio interface between both outdoor units.

- RF_TO_LOCAL:

  - Returning the header information of the local site back to the local site on the radio interface between both outdoor units.

- IF_TO_REMOTE:

  - Returning the header information of the remote site back to the remote site on the intermediate frequency interface between local indoor unit and outdoor unit.

- IF_TO_LOCAL:

  - Returning the header information of the local site back to the local site on the intermediate frequency interface between local indoor unit and outdoor unit.

- NONE:

## PolarizationType

Contains Enumeration Literals:

- HORIZONTAL:

- VERTICAL:

- NOT_YET_DEFINED:

## RadioSignalIdDatatypeType

Contains Enumeration Literals:

- INTEGER:

- STRING:

- NOT_YET_DEFINED:

## XLevelThresholdSecondKindType

According to ETSI EN 301 129 V1.1.2 (1999-05) chapter B.1.1

Contains Enumeration Literals:

- RLTS:

- TLTS:

- NOT_YET_DEFINED: